KCSv3 protocol – Ansi String

String format command can use for “TCP Server/TCP Client/RS232/UCP Server/UDP Client” work mode.

- Read all digital output state

Send: RELAY-STATE-255

Feedback: RELAY-STATE-255, D7,D6,D5,D4,D3,D2,D1,D0,OK

D7,D6,D5,D4,D3,D2,D1,D0 are “decimal” number, every data convert to binary, bit ”1” is ON, bit “0” is OFF.

if use KC868-A64 , it have 64 digital output, every byte have 8 bit, every bit mean every digital output state, so KC868-A64 have 8 bytes. Feedback format is RELAY-STATE-255,D7,D6,D5,D4,D3,D2,D1,D0

For example: feedback is  RELAY-STATE-255,0,0,0,0,0,0,5,128,OK

D7=(0)dec=(00000000)b   output (64-57)       means:output57—64: OFF

D6=(0)dec=(00000000)b   output (56-49)       means:output49—56: OFF

D5=(0)dec=(00000000)b   output (48-41)       means:output41—48: OFF

D4=(0)dec=(00000000)b   output (40-33)       means:output33—40: OFF

D3=(0)dec=(00000000)b   output (32-25)       means:output25—32: OFF

D2=(0)dec=(00000000)b   output (24-17)       means:output17—24: OFF

D1=(0)dec=(00000101)b   output (16-9)        means: output9,11: ON others: OFF

D0=(128)dec=(10000000)b  output (8-1)        means: output8: ON others: OFF

2. auto feedback one digital input state

feedback: RELAY-INPUT-255,1,1     DI1=true

feedback: RELAY-INPUT-255,1,0     DI1=false

feedback: RELAY-INPUT-255,2,1     DI2=true

feedback: RELAY-INPUT-255,2,0     DI2=false

3. Read all digital input state

send：RELAY-GET_INPUT-255

feedback：RELAY-GET_INPUT-255, D7,D6,D5,D4,D3,D2,D1,D0,OK

D7,D6,D5,D4,D3,D2,D1,D0 are “decimal” number, every data convert to binary, bit ”0” is trigger, bit “1” is not trigger.

if use KC868-A64 , it have 64 digital input, every byte have 8 bit, every bit mean every digital input state, so KC868-A64 have 8 bytes. Feedback format is RELAY-STATE-255,D7,D6,D5,D4,D3,D2,D1,D0

For example: feedback is  RELAY-STATE-255,255,255,255,255,255,255,255,127,OK

D7=(255)dec=(11111111)b   output (64-57)       means:input57—64: not trigger

D6=(255)dec=(11111111)b   output (56-49)       means:input49—56: not trigger

D5=(255)dec=(11111111)b   output (48-41)       means:input41—48: not trigger

D4=(255)dec=(11111111)b   output (40-33)       means:input33—40: not trigger

D3=(255)dec=(11111111)b   output (32-25)       means:input25—32: not trigger

D2=(255)dec=(11111111)b   output (24-17)       means:input17—24: not trigger

D1=(255)dec=(11111111)b   output (16-9)        means: input9,11: trigger    others: not trigger

D0=(127)dec=(01111111)b  output (8-1)          means: input8: trigger       others: not trigger

4. Read one channel of ADC (analog input) state

RELAY-GET_ADC-255,id

send：RELAY-GET_ADC-255,1

feedback：RELAY-GET_ADC-255,1,100,OK     means: channel-1 ADC value=100

"id" means: 1-MAX channel number.

“100” is ADC original acquisition value. Range: 0-4095 -> dc 0-5v input

5. Read one channel of DAC (analog output) state

RELAY-GET_DAC-255,id

send：RELAY-GET_DAC-255,1

feedback：RELAY-GET_DAC-255,1,100,OK    means: channel-1 DAC value=100

"id" means: 1-MAX channel number.

“100” is DAC output value. Range: 0-255 -> dc 0-10v output

6. Read one channel of digital output state

RELAY-READ-255,id

send：RELAY-READ-255,2       means: turn digital output-2 OFF

feedback：RELAY-READ-255,2,0,OK

"id" means: 1-MAX channel number.

“0” is OFF, “1” is ON.

7. Set ON/OFF one channel of digital output

RELAY-SET-255,id,state

send：RELAY-SET-255,1,0      measn: turn OFF digital output-1

feedback：RELAY-SET-255,1,0,OK

"id" means: 1-MAX channel number.

“state” means: “0” is OFF, “1” is ON.

8. Set one channel of DAC output

RELAY-SET_DAC-255,id,state

send：RELAY-SET_DAC-255,1,200

feedback：RELAY-SET_DAC-255,1,200,OK

"id" means: 1-MAX channel number.

“state” range: 0-255 -> dc 0-10v output

9. auto feedback one DAC output state

Feedback: RELAY-GET_DAC-255,1,130,OK

1: channel one   130: output value   output value range: 0--255

10. auto feedback Analog input (ADC) value

Feedback: RELAY-GET_ADC-ALL-255,0,0,0,0,OK

0,0,0,0 are 4 channel analog input value.

Note: after enabled “auto send ADC values” option in KCS “SYSTEM” webpage. Will auto feedback all analog input value every 5 seconds.

Feedback analog input RAW value:

If ADC by ESP32: range: 0--4095

If ADC by ADS1115: range: 0--26489

11. Set ON/OFF/TOGGLE for any multi channel of digital output

RELAY-SET_MULTI-255,

D23,D22,D21,D20,D19,D18,D17,D16,D15,D14,D13,D12,D11,D10,D9,D8,D7,D6,D5,D4,D3,D2,D1,D0

if use KC868-A64 , it have 64 digital output, every byte have 8 bit, every bit mean every digital output state, so KC868-A64 have 8 bytes. We will use ON/OFF/TOGGLE for these, so total need 8*3=24 bytes.

(D23,D22,D21,D20,D19,D18,D17,D16) use for ON command

(D15,D14,D13,D12,D11,D10,D9,D8) use for OFF command

(D7,D6,D5,D4,D3,D2,D1,D0) use for TOGGLE command

D23,D22,D21,D20,D19,D18,D17,D16,D15,D14,D13,D12,D11,D10,D9,D8,D7,D6,D5,D4,D3,D2,D1,D0

are “decimal” number, every data convert to binary, bit ”1” is effective , bit “0” is ineffective.

For example:

send：RELAY-SET_MULTI-255,0,0,0,0,0,0,0,128,0,0,0,0,0,0,0,64,0,0,0,0,0,0,0,32

feedback：RELAY-SET_MULTI-255,OK

D16=(128)dec=(10000000)b  means: turn ON output-8

D8=(64)dec=(01000000)b    means: turn OFF output-7

D0=(32)dec=(00100000)b    means: TOGGLE output-6

So send this command, will turn ON output-8,turn OFF output-7, TOGGLE output-6 simultaneously.

12. Set ON/OFF multi channel of digital output

RELAY-SET_ALL-255,D7,D6,D5,D4,D3,D2,D1,D0

send：RELAY-SET_ALL-255,0,0,0,0,0,0,5,128

feedback：RELAY-SET_ALL-255,0,0,0,0,0,0,5,128,OK

if use KC868-A64 , it have 64 digital output, every byte have 8 bit, every bit mean every digital output state, so KC868-A64 have 8 bytes. Feedback format is RELAY-SET_ALL-255,D7,D6,D5,D4,D3,D2,D1,D0,OK

For example: send command is  RELAY-SET_ALL-255,0,0,0,0,0,0,5,128

D7=(0)dec=(00000000)b   output (64-57)       means:output57—64: OFF

D6=(0)dec=(00000000)b   output (56-49)       means:output49—56: OFF

D5=(0)dec=(00000000)b   output (48-41)       means:output41—48: OFF

D4=(0)dec=(00000000)b   output (40-33)       means:output33—40: OFF

D3=(0)dec=(00000000)b   output (32-25)       means:output25—32: OFF

D2=(0)dec=(00000000)b   output (24-17)       means:output17—24: OFF

D1=(0)dec=(00000101)b   output (16-9)        means: output9,11: ON others: OFF

D0=(128)dec=(10000000)b  output (8-1)        means: output8: ON others: OFF

So use this command will turn ON output8,9,11 turn OFF others simultaneously.

13. Set all channels of digital output ON

send：RELAY-AON-255,1,1

feedback：RELAY-AON-255,1,1,OK

14. Set all channels of digital output OFF

send：RELAY-AOF-255,1,1

feedback：RELAY-AOF-255,1,1,OK

15. Actively report information

feedback：RELAY-NOTIFY-255,id,state

"id" means: 1-MAX channel number.

“state” means: “0” is OFF, “1” is ON.

Any digital output changed STATE by any way , it will actively report message.

16. Toggle state of digital output

Send:RELAY-KEY-255,id,1

Feedback:RELAY-KEY-255,id,1,OK

"id" means: 1-MAX channel number.

change the status of one digital output, such as your output is ON, when send this command , it will be OFF. if your output is OFF, when send this command , it will be ON.

17. Send one RF signal have learned

RELAY-RF-RUN-255,id

Send: RELAY-RF-RUN-255,1

Feedback: RELAY-RF-RUN-255,1,OK

18. Set ON/OFF of buzzer

RELAY-SET-BEEP,state

send：RELAY-SET-BEEP,1         measn: turn ON buzzer

feedback：RELAY-SET-BEEP,1,OK

send：RELAY-SET-BEEP,0         measn: turn OFF buzzer

feedback：RELAY-SET-BEEP,0,OK

19. IR commands:

Learn IR (MAX support 192 IR signals):

send:  RELAY-IR-LEARN-255,1,2    // begin learn IR signal ID=1, send by IR tube-2 (note: AG8 controller have 8 CH IR tube)

feedback: RELAY-IR-LEARN-255,1,OK  // success

feedback: RELAY-IR-LEARN-255,1,TIMEOUT  // timeout

send: RELAY-IR-LEARN-255,200  // if send IR exceeding the maximum IR ID(MAX=192)

feedback: RELAY-IR-LEARN-255,200,ERROR  // error

Send IR:

send: RELAY-IR-RUN-255,1  // send IR signal ID=1

feedback: RELAY-IR-RUN-255,1,OK

Delete IR:

send: RELAY-IR-DEL-255,1  // delete learned IR signal ID=1

feedback: RELAY-IR-DEL-255,1,OK

---------------------------------------------------------------

IR triggered and auto feedback: (ONLY use for NEC and RC5 IR code)

feedback:

RELAY-GET_IR-255,1,1,OK  // trigger

RELAY-GET_IR-255,1,0,OK  // not trigger